LiftMaster	Distributed by ZeroDocs.com

SECTION 11 12 33 – PARKING GATES

- GENERAL

  - SUBMITTALS

    - Action Submittals:

      - Product Data: Manufacturer’s descriptive data and product attributes.

    - Closeout Submittals:

      - Operation and Maintenance Data.

  - QUALITY ASSURANCE

    - Installer Qualifications: Firm specializing in work of this Section, with minimum [2] [__] years’ experience.

  - WARRANTY

    - Manufacturer’s warranties against material and manufacturing defects:

      - Cabinet: 10 years.

      - Electrical and mechanical components: 2 years.

- PRODUCTS

  - MANUFACTURERS

    - Contract Documents are based on products by LiftMaster. www.LiftMaster.com

    - Substitutions: [Refer to Division 01.] [Not permitted.]

  - MANUFACTURED UNITS

    - Parking Gates:

      - Model: MAS.

      - Operation: Gear reduced.

      - Meet UL 325.

      - Rated duty cycle: Maximum 10,000 per day.

      - Opening speed: 0.9 seconds.

      - Battery backup capable.

      - Motor: 24 VDC, continuous duty, prewired, listed by Underwriters Laboratories.

      - Controls: Sequenced access management system.

      - Cabinet: Polyethylene.

      - Aluminum 8 foot arm with yellow padding.

      - Clutch.

      - Retro-reflective photo eye.

      - [One button DIP remote control.] [Three button DIP remote control.] [Two button rolling code Security+ 2.0 remote control.] [Four button rolling code Security+ 2.0 remote control.]

      - Red/green traffic light.

      - Low power loop detector.

      - Commercial access control receiver.

      - Heater.

      - Timer-to-close module.

      - High-capacity connected access portal.

      - Two door connected access portal.

- EXECUTION

  - INSTALLATION

    - Install in accordance with manufacturer’s instructions.

  - CLOSEOUT ACTIVITIES

    - Test and adjust operators for proper operation.

    - Demonstration: Demonstrate operation and programming of operators to Owner.

END OF SECTION