AMERISTAR® PERIMETER SECURITY USA INC.

ECHELON II® - Heavy Industrial Aluminum Ornamental Fence System – Internally Secured

Construction Specification - SECTION 32 31 19

PART 1 - GENERAL

1.01 WORK INCLUDED

The contractor shall provide all labor, materials and appurtenances necessary for installation of the industrial ornamental aluminum fence system defined herein at (specify project site).

1.02 RELATED WORK

Section _ _ _ - Earthwork

Section _ _ _ - Concrete

1.03 SYSTEM DESCRIPTION

The manufacturer shall supply a total industrial ornamental aluminum fence system of the Ameristar® Echelon II® (specify Classic™, Majestic™, Genesis™ or Invincible™) design. The system shall include all components (i.e., pickets, rails, posts, gates and hardware) required.

1.04 QUALITY ASSURANCE

The contractor shall provide laborers and supervisors who are thoroughly familiar with the type of construction involved and materials and techniques specified.

1.05 REFERENCES

- ASTM B117 - Practice for Operating Salt-Spray (Fog) Apparatus.

- ASTM B221 - Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles and Tubes.

- ASTM D523 - Test Method for Specular Gloss.

- ASTM D822 - Practice for Conducting Tests on Paint and Related Coatings and Materials using Filtered Open-Flame Carbon-Arc Light and Water Exposure Apparatus.

- ASTM D1654 - Test Method for Evaluation of Painted or Coated Specimens Subjected to Corrosive Environments.

- ASTM D2244 - Test Method for Calculation of Color Differences from Instrumentally Measured Color Coordinates.

- ASTM D2794 - Test Method for Resistance of Organic Coatings to the Effects of Rapid Deformation (Impact).

- ASTM D3359 - Test Method for Measuring Adhesion by Tape Test.

1.06 SUBMITTAL

The manufacturer’s submittal package shall be provided prior to installation.

1.07 PRODUCT HANDLING AND STORAGE

Upon receipt at the job site, all materials shall be checked to ensure that no damage occurred during shipping or handling. Materials shall be stored in such a manner to ensure proper ventilation and drainage, and to protect against damage, weather, vandalism, and theft.

PART 2 - MATERIALS

2.01 MANUFACTURER

The industrial ornamental aluminum fence system shall conform to Ameristar Echelon II, (specify Classic, Majestic, Genesis or Invincible) (specify 2-Rail, 3-Rail, 4-Rail, 3-rail with rings, or 4-rail with rings) style manufactured by Ameristar Fence Products, Inc., in Tulsa, Oklahoma.

2.02 MATERIAL

A.  Aluminum material for fence framework (i.e., tubular pickets, rails, and posts) shall conform to the requirements of ASTM B221. The aluminum extrusions for posts and rails shall be Alloy and Temper Designation 6005-T5. The aluminum extrusions for pickets shall be Alloy and Temper Designation 6063-T52.

B.  The manufactured framework shall be subjected to the Ameristar thermal stratification coating process (high-temperature, in-line, multi-stage, and multi-layer) including, as a minimum, a six-stage pretreatment/wash, and an electrostatic spray application of a polyester finish. The topcoat shall be a “no-mar” TGIC polyester powder coat finish with a minimum thickness of 2 mils (0.0508mm). The color shall be (specify Black, Bronze, White, or Desert Sand). The stratification-coated framework shall be capable of meeting the performance requirements for each quality characteristic shown in Table 2.

C.  Material for fence pickets shall be 1" square x 0.062" thick (.125” wall for Invincible) extruded tubing. The cross-sectional shape of the rails shall conform to the manufacturer's ForeRunner™ design with outside cross-section dimensions of 1.75" square. The top wall and internal web of the rail shall be 0.070" thick; the sidewalls shall be 0.070" thick for superior vertical load strength. Picket holes in the ForeRunner rail shall be spaced 4.715” o.c., except for Invincible style 6’ long, which shall be, spaced 4.98” o.c. Picket retaining rods shall be 0.125” diameter galvanized steel. Fence posts and gate posts shall meet the minimum size requirements of Table 1. High quality PVC grommets shall be supplied to seal all picket-to-rail intersections.

D.  Bracket to rail attachments shall be made using specially designed one-way tamperproof security nuts with carriage bolt. Bracket to post connections shall be made using self-drilling hex-head screws.

E.  Aluminum castings shall be used for all rings, post caps, finials, and miscellaneous adornments.

2.03 FABRICATION

A.  Pickets, rails, and posts shall be pre-cut to specified lengths. ForeRunner rails shall be pre-punched to accept pickets.

B.  The rail inner slide shall be fully inserted into the rail outer channel to form the raceway for the internal retaining rod. Grommets shall be inserted into the pre-punched holes in the rails, and pickets shall be inserted through the grommets so that pre-drilled picket holes align with the internal raceway of the two-part ForeRunner rails. (Note:  This can best be accomplished by using an alignment template). Retaining rods shall be inserted into each ForeRunner rail so that they pass through the pre-drilled holes in each picket, thus completing the panel assembly.

C.  Completed panels shall be capable of supporting a 300 lb. load (applied at midspan) without permanent deformation. Panels shall be biasable to a 25% change in grade.

D.  Gates shall be fabricated using 1.75” sq. reinforced ForeRunner rail material, 2” sq. x .250” gate ends, and 1” sq. x .125” pickets. All rail and upright intersections shall be joined by welding. All picket and rail intersections shall be joined by welding.

PART 3 - EXECUTION

3.01 PREPARATION

All new installation shall be laid out by the contractor in accordance with the construction plans.

3.02 FENCE INSTALLATION

Fence post shall be spaced according to Table 3, plus or minus ½”. For installations that must be raked to follow sloping grades, the post spacing dimension must be measured along the grade. Fence panels shall be attached to posts with brackets supplied by the manufacturer. Posts shall be set in concrete footers having a minimum depth of 36” (Note:  In some cases, local restrictions of freezing weather conditions may require a greater depth). The “Earthwork” and “Concrete” sections of this specification shall govern material requirements for the concrete footer. Posts setting by other methods such as plated posts or grouted core-drilled footers are permissible only if shown by engineering analysis to be sufficient in strength for the intended application.

3.03 FENCE INSTALLATION MAINTENANCE

When cutting/drilling rails or posts adhere to the following steps to seal the exposed surfaces; 1) Remove all metal shavings from cut area. 2) Apply custom finish paint matching fence color. Failure to seal exposed surfaces per steps 1& 2 above will negate warranty. Ameristar spray cans or paint pens shall be used to prime and finish exposed surfaces; it is recommended that paint pens be used to prevent overspray. Use of non-Ameristar parts or components will negate the manufactures’ warranty.

3.04 GATE INSTALLATION

Gate posts shall be spaced according to the manufacturers’ gate drawings, dependent on standard out-to-out gate leaf dimensions and gate hardware selected. Type and quantity of gate hinges shall be based on the application; weight, height, and number of gate cycles.  The manufacturers’ gate drawings shall identify the necessary gate hardware required for the application. Gate hardware shall be provided by the manufacture of the gate and shall be installed per manufacturer’s recommendations.

3.05 CLEANING

The contractor shall clean the jobsite of excess materials; post-hole excavations shall be scattered uniformly away from posts.

| Table 1 – Minimum Sizes for Echelon II Posts |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Fence Posts |  | Panel Height |  |  |  |
| 2-1/2” x 2-1/2” x .080” Alum. w/ reinforced web |  | Up to & Including 6’ Height |  |  |  |
| 3” x 3” x .120” Alum. |  | Over 6’ Up to & Including 8’ Height |  |  |  |
| 4” x 4” x .250” Alum. |  | Over 8’ Height Up to 10’ |  |  |  |
| Gate Leaf | Gate Height |  |  |  |  |
|  | Up to & Including 4’ |  | Over 4’ Up to & Including 6’ | Over 6’ Up to & Including 8’ | Over 8’ Up to & Including 10’ |
| Up to 4’ | 3” x 3” x .120” Alum. |  | 4” x 4” x .250 Alum. or 3” x 12 Ga. steel | 4” x 11 Ga. steel | 4” x 11 Ga. steel |
| 4’1” to 6’ | 4” x 4” x .250 Alum. or 3” x 12Ga. steel |  | 3” x 12 Ga. steel | 4” x 11 Ga. steel | 4” x 11 Ga. steel |
| 6’1” to 8’ | 4” x 11 Ga. steel |  | 4” x 11 Ga. steel | 4” x 11 Ga. steel | 6” x 3/16” steel |
| 8’1” to 10’ | 4” x 11 Ga. steel |  | 4” x 11 Ga. steel | 6” x 3/16” steel | 6” x 3/16” steel |
| 10’1” to 12’ | 4” x 11 Ga. steel |  | 6” x 3/16” steel | 6” x 3/16” steel | 6” x 3/16” steel |
| 12’1” to 14’ | 6” x 3/16” steel |  | 6” x 3/16” steel | 6” x 3/16” steel | 6” x 3/16” steel |

| Table 2 – Coating Performance Requirements |  |  |
| --- | --- | --- |
| Quality Characteristics | ASTM Test Method | Performance Requirements |
| Adhesion | D3359 – Method B | Adhesion (Retention of Coating) over 90% of test area (Tape and knife test). |
| Corrosion Resistance | B117 & D1654 | Corrosion Resistance over 1,000 hours (Scribed per D1654; failure mode is accumulation of 1/8” coating loss from scribe or medium #8 blisters). |
| Impact Resistance | D2794 | Impact Resistance over 60 inch lb. (Forward impact using 0.625” ball). |
| Weathering Resistance | D822 D2244, D523 (60˚ Method) | Weathering Resistance over 1,000 hours (Failure mode is 60% loss of gloss or color variance of more than 3 delta-E color units). |

| Table 3 – Echelon II – Post Spacing By Bracket Type |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Span | For INVINCIBLE® 8’ Nominal (91.25” Rail) |  | For CLASSIC, GENESIS, & MAJESTIC 8’ Nominal (92.625” Rail) |  |  |  |  |  |
| Post Size | 2-1/2” | 3” | 2-1/2” | 3” | 2-1/2” | 3” | 2-1/2” | 3” |
| Bracket Type | Industrial Flat Mount (BB301) |  | Industrial Universal (BB302) | Industrial Universal (BB303) | Industrial Flat Mount (BB301) |  | Industrial Swivel (BB304)* |  |
| Post Settings ± ½” O.C. | 94-1/2” | 95” | 96” | 96.5” | 96” | 96-1/2” | *97.5” | *98” |
| Span | For INVINCIBLE® 6’ Nominal (71.375” Rail) |  | For CLASSIC, GENESIS, & MAJESTIC 6’ Nominal (67.75” Rail) |  |  |  |  |  |
| Post Size | 2-1/2” | 3” | 2-1/2” | 3” | 2-1/2” | 3” | 2-1/2” | 3” |
| Bracket Type | Industrial Flat Mount (BB301) |  | Industrial Universal (BB302) | Industrial Universal (BB303) | Industrial Flat Mount (BB301) |  | Industrial Swivel (BB304)* |  |
| Post Settings ± ½” O.C. | 75” | 75.5” | 71.5” | 72” | 71.5”” | 72” | *73” | *73.5” |
| *Note: When using BB304 swivel brackets on either or both ends of a panel installation, care must be taken to ensure the spacing between post and adjoining pickets meets applicable codes. This will require trimming one or both ends of the panel. |  |  |  |  |  |  |  |  |